COMPUTER GRAPHICS.

PROJECT DOCUMENTATION.

14-STOREY NYLA AIR BNB.

DESIGNED USING ARCHICAD.

GROUP MEMBERS.

- GRACE MUINDI BIT/2024/59311.

- EMMANUELI MURIUKI BIT/2024/61120.

- RISPER CHEPNGENO BIT/2024/55500.

- EMILY ATIENO BIT/2024/45448.

- ABUNA RICHARD BIT/2024/73577.

- KELLY MAROA BIT/2024/49929.

- JAMES MAM BIT/2024/56261.

- AGNES NUNGARI BIT/2024/54977.

- TONNY KIPCHUMBA BIT/2024/54787.

- EDWARD YOHANA BIT/2024/53391.

- ARSHLEY AGATHA BIT/2024/60044.

PROJECT OVERVIEW.

Project Title: 14-Storey Nyla Air BnB.

Software used: Archicad.

Total Floors: 13-Storey + Roof.

Structure Type: Reinforced Concrete Frame.

Design Methodology: Building Information Modelling (BIM).

This project involves the design, modelling, and documentation of a fourteen-storey building using BIM technology. The model integrates architectural, structural, plumbing, electrical, and networking ICT systems within a coordinated 3D environment.

NYLA BNB PROJECT

Integrated Building Services & Infrastructure Technical Documentation

(14-Storey Commercial Residential Development).

1. Introduction.

The Nyla BnB is a 14-storey commercial residential development designed to provide modern, high-quality hospitality services within an urban setting. Beyond architectural aesthetics, the success of the building depends heavily on the efficiency and integration of its technical service systems.

This documentation presents a comprehensive overview of the building’s:

- Information and Communication Technology (ICT) infrastructure

- Electrical distribution system

- Water supply and plumbing system

The report demonstrates how these systems are coordinated through a centralized vertical service riser to achieve reliability, safety, sustainability, and guest satisfaction. The project was modeled and coordinated using ArchiCAD 28, ensuring a Building Information Modeling (BIM) approach to service integration.

2. Project Vision and Objectives.

The Nyla BnB project was developed with both technical precision and human comfort in mind. The key objectives guiding the infrastructure design include:

2.1 Infrastructure Reliability.

A strong and centralized vertical backbone system was designed to distribute ICT, electrical, and plumbing services across all 14 floors. This ensures:

- Simplified maintenance.

- Reduced system downtime.

- Efficient service coordination.

2.2 Safety and Compliance.

All installations follow modern building and electrical safety principles, including:

- Circuit isolation through floor-based distribution boards.

- Proper earthing and overload protection.

- Pressure-regulated water supply systems.

2.3 Guest-Centered Comfort.

The building prioritizes guest convenience through:

- Full wireless coverage in every room

- Ergonomically placed sockets and switches

- Consistent lighting and uninterrupted water supply

2.4 Sustainability and Continuity.

To ensure long-term service reliability, the project incorporates:

- Underground water storage.

- Overhead gravity-fed tanks.

- Efficient electrical zoning.

- Optimized pump and pressure systems.

3. ICT & Networking Infrastructure.

Reliable internet connectivity is essential in modern hospitality environments. The Nyla BnB integrates a high-speed structured networking system designed for seamless digital access.

3.1 Vertical Fiber Backbone.

A dedicated service shaft (riser), located adjacent to the stairwell, houses fiber optic cables extending from the ground floor to the roof. This vertical backbone acts as the digital highway of the building.

The backbone ensures:

- High-speed data transmission.

- Scalability for future upgrades.

- Organized cable management.

3.2 Main Distribution Frame (MDF).

The MDF is positioned on the ground floor and serves as the central control hub. It:

- Receives internet service from the external ISP.

- Houses routers and core switches.

- Distributes fiber uplinks to each floor.

3.3 Intermediate Distribution Frames (IDFs).

Each floor contains an IDF cabinet that manages horizontal data distribution to individual rooms using Cat6a cabling. This floor-level segmentation enhances troubleshooting and performance management.

3.4 Wireless Access Points (WAPs).

Wireless Access Points are strategically installed along corridors to provide:

- Seamless roaming across floors.

- Full coverage inside guest rooms.

- Stable multi-device connectivity.

This design ensures every guest experiences uninterrupted connectivity for work and leisure.

4. Electrical System Design.

The electrical system was designed to prioritize safety, accessibility, and operational efficiency.

4.1 Main Power Entry.

Electrical power enters the building via an underground supply line and connects to a central meter box on the ground floor.

4.2 Distribution Strategy.

Each floor is equipped with a dedicated Distribution Board (DB). This approach:

- Isolates faults to specific floors.

- Prevents total building outages.

- Simplifies maintenance.

- Circuit breakers and overload protection devices ensure safe operation.

4.3 Lighting System.

Guest rooms are fitted with centrally positioned lighting points. Switches are ergonomically installed at approximately 1200 mm above floor level for ease of use.

The lighting layout ensures:

- Balanced illumination.

- Safe wiring concealment.

- Energy efficiency.

4.4 Power Outlets.

Twin 13A socket outlets are installed at 300 mm above finished floor level. Their positioning allows convenient charging of:

- Mobile phones.

- Laptops.

- Personal electronic devices.

This arrangement reflects thoughtful planning aligned with modern user behavior.

5. Water Supply & Plumbing System.

Water reliability is essential in hospitality facilities. The Nyla BnB integrates a dual-supply system to guarantee continuous availability.

5.1 Municipal Water Intake.

Municipal water enters the building through a main supply line and is directed into a large underground storage tank located within the ground-floor pump room.

5.2 Pumping System.

A close-coupled end suction pump lifts water vertically through the building’s 42-meter height via the Vertical Riser system. The pump ensures adequate pressure across all floors.

5.3 Vertical Water Riser.

The Vertical Riser serves as the main hydraulic spine of the building. It:

- Aligns consistently through all floors.

- Distributes water horizontally to bathrooms.

- Ensures pressure consistency.

5.4 Bathroom Fixture Connections.

Each guest bathroom includes:

- Water Closet (WC).

- Wash Hand Basin (WHB).

- Shower system.

Water is distributed from the corridor supply spine into each fixture through branch piping.

5.5 Overhead Tanks (Gravity Backup).

Two overhead storage tanks are installed at roof level. During power outages:

- Water flows through gravity.

- Service continuity is maintained.

- Pressure fluctuations are minimized.

- This redundancy ensures 24/7 availability.

6. BIM Implementation Using ArchiCAD 28.

The entire building was modeled using ArchiCAD 28, applying:

- Story duplication for consistent floor layouts.

- Layer separation for service coordination.

- Object tools for accurate MEP component placement.

- Vertical alignment tools for riser consistency.

The BIM approach allows:

- Clash detection between systems.

- Accurate documentation.

- Improved lifecycle management.

7. Conclusion.

The Nyla BnB 14-storey development successfully integrates ICT, electrical, and plumbing systems into a coordinated and efficient infrastructure model.

Through the use of a centralized vertical riser system and BIM-based modeling in ArchiCAD 28, the project achieves:

- Infrastructure reliability.

- Operational safety.

- Guest comfort.

- Sustainable resource management.

The building stands as a well-coordinated hospitality facility that meets modern technological, environmental, and safety standards.

8. References (APA 7th Edition).

Ching, F. D. K., & Winkel, S. R. (2018). Building systems for interior designers (3rd ed.). Wiley.

Eastman, C., Teicholz, P., Sacks, R., & Liston, K. (2011). BIM handbook: A guide to building information modeling for owners, managers, designers, engineers and contractors (2nd ed.). Wiley.

Grondzik, W. T., & Kwok, A. G. (2019). Mechanical and electrical equipment for buildings (13th ed.). Wiley.

International Code Council. (2021). International building code (IBC). ICC.

National Fire Protection Association. (2020). NFPA 70: National electrical code. NFPA